Tulip Medical

Quality System Procedure

QSP 4.2-5

Medical Device File

Approvals

VP Regulatory Affairs	Date

Regulatory Affairs Coordinator	Date

Document Control Administrator	Date

| Rev | Date | Description of Change |
| --- | --- | --- |
| 0 | 09/11/2018 | Initial release. |
| 1 | 3/14/2024 | Added “Including the safety and efficacy of the device” to section 9.1.2 |
| 2 | 5/23/2025 | 1.0 Rephrased purpose to comply with MDR 3.0 Added reference to the MDR 8.1 Removed MDD Annex I section 9.2.1 Rephrased from MDD to align with MDR compliance |

- PURPOSE

ISO 13485:2016, Clause 4.2.3 defines the requirements for the Medical Device File (MDF). This procedure describes these requirements and incorporates the applicable provisions of EN ISO 13485:2016 in alignment with the European Medical Device Regulation (MDR) 2017/745

- SCOPE

This procedure pertains to the development and maintenance of Medical Device Files for Tulip Medical Product Families.

- REFERENCE DOCUMENTS

| Regulation (EU) 2017/745 on medical devices (MDR) |  |
| --- | --- |
| 21 Code of Federal Regulations 820.818 Device Master Record |  |
| Form 4.2-5 | Medical Device File |
| Form 7.3-2-1 | Device Master Record Index |
| QSP 4.2-1 | Document Control |
| QSP 7.3-2 | Device Master Record |
| QSP 9.1-3 | Development of Technical File |

- DEFINITIONS

Containing Or Referencing Documents - The medical device file includes the documents necessary to show both kinds of conformity: conformity to the standard and conformity to regulatory requirements. The documents, or at least copies of them, could be collected into the medical device file. There was a time when they would all be in a three ring binder. Today they could be collected in an electronic file. Alternately, there could be an electronic file with pointers to the documents. Both methods are satisfactory, but the manufacturer should decide to use only one.  It would be useful, regardless of the method selected, to identify each document by its role. A document could satisfy a requirement of the standard, a regulatory requirement, or both.

Determining Families - To set up the medical device file for a family the first step determines which devices are in the family. One method uses a matrix with potential family members listed in the first column. Subsequent columns include safety, intended use, and function. The final column list the family name for each device in the first column. The rows list the basic design and performance characteristics of the device. This method identifies devices that share the same basic characteristics.

Documents To Demonstrate Conformity - The medical device file includes documents that generate conformity to the standard and to regulatory requirements. One of the requirements of the standard asks the manufacturer to  determine the manufacturer’s roles each regulatory region, identify the requirements, and establish the necessary processes. As a result, there are two sets of requirements to consider.  The requirements from the standard are listed in the clause. Additionally, regulatory regions create specific requirements. For the EU, EN ISO 13485:2016 is an example. Another source is the regulatory requirements cited in the MDSAP Audit Document. This identifies the requirements in the MDSAP geographic areas: Australia, Brazil, Canada, Japan, and the US. A subsequent section looks at these requirements.

Management Representative – The member of Tulip Medical whose cross-functional role encompasses the development and monitoring of the quality management system to ensure accuracy and effectiveness of all Quality System Procedures (QSPs).

Medical Device Family - The medical device file applies to either each medical device type or to medical device family. The standard doesn’t define medical device type, but ISO 13485:2016 clause 3.12 has a definition for medical device family. Medical Device Family means a group of medical devices manufactured by or for the same organization and having the same basic design and performance characteristics related to safety, intended use, and function.

Medical Device Single Audit Program (MDSAP) – The Medical Device Single Audit Program (MDSAP) allows an MDSAP recognized Auditing Organization to conduct a single regulatory audit of a medical device manufacturer that satisfies the relevant requirements of the regulatory authorities participating in the program.

One Or More Files - It appears the standard would allow more than one file for the same device type or device family. This is not usually a good idea; there is advantage in keeping all of the information about a device type or device family together.

- ABBREVIATIONS

MDF – Medical Device File

MDSAP – Medical Device Single Audit Program

MR – Management Representative

- RESPONSIBILITIES

The Management Representative (MR) is responsible for maintaining this procedure in a current state.

- THE SIX SPECIFIC REQUIREMENTS FROM THE STANDARD

  - General Description

    - Clause 4.2.3.a says that the medical device file includes a general description of the medical device, the intended use/purpose, and the labeling (including any instructions for use).  The general description describes the device, and should include the device characteristics, method of operation, functional requirements, etc. For a device family the general description should include the basic design and performance characteristics related to safety, intended use, and function that define the family. The intended use or purpose is part of the general information about the device, but often overlaps with regulatory requirements for an intended use statement. In many regulatory areas, labeling is important and potentially has many meanings. It will certainly include the label that goes on the device package or the device itself. Notice that the requirement here is that labeling includes any instructions for use.

  - Product Specifications

    - Clause 4.2.3.b requires specifications for the product. Think of this as the specific requirements for the device in contrast with the general description. Often these are the product specifications included in the final acceptance criteria. These are the characteristics to verify before the device is released for distribution.

  - Specifications and Procedures

    - Clause 4.2.3.c includes the specifications or procedures for manufacturing, packaging, storage, handling, and distribution.  This is a long list of requirements for procedures or specifications. Typically, the manufacturer implements procedures for each of these characteristics.  Manufacturing procedures include all of the procedures and work instructions for production, product verification, process validation, identification, inspection status, etc.  Packaging procedures cover device packaging and may include packaging that protects the device storage or transit. It could also include sterile barrier systems.

    - Storage procedures explain the requirements for the device while it is in the stockroom. For example, a device may require storage under controlled conditions such as temperature or humidity. Handling procedures may also include specific requirements such as the use of specific equipment.  Distribution procedures cover the device while in the distribution channel; the period of time from leaving the manufacturer until arriving at the customer’s site.

  - Measures and Monitoring

    - Clause 4.2.3.d requires procedures for measuring and monitoring. ISO 9000:2015 includes definitions of measuring and monitoring. Measurement means a process to determine a value. [ISO 9000:2015, 3.11.4] Monitoring means determining the status of a system, a process, a product, a service, or an activity. [ISO 9000:2015, 3.11.3] Often, measurement is associated with product verification activities. Monitoring is often associated with process control. For example, in SPC, the operator would measure a product characteristic on the items in the subgroup, plot them on the charts, and use the information to monitor whether the process is in control.

  - Installation Requirements – Not Applicable

    - In accordance with ISO 13485:2016 Clause 7.5.3.

  - Servicing

    - ISO 13485:2016 Clause 4.2.3.f covers servicing.  However, some devices don’t require servicing.  Servicing could be done internally, under contract to the manufacturer, or by a third party.  If servicing is required, then the procedures include service record analysis. Be sure to cover all the requirements in Clause 7.5.4.

- ADDITIONAL REQUIREMENTS

  - MDSAP Audit Tasks – MDSAP is divided into processes and further subdivided into tasks. The tasks have reference to requirements, so it is interesting to look at the tasks that refer to ISO 13485:2016, clause 4.3.2.

  - Process: Design and Development Task 7 – Review medical device specifications to confirm that design and development outputs are traceable to and satisfy design input requirements. Verify that the design and development outputs essential for the proper functioning of the medical device have been identified. Outputs include, but are not limited to, device specifications, specifications for the manufacturing process, specifications for the sterilization process (if applicable), the quality assurance testing, and device labeling and packaging.

  - Process: Design and Development Task 13 – Verify that design and development changes were controlled, verified (or where appropriate validated), and approved prior to implementation. Confirm that any new risks associated with the design change have been identified and [reduced] to the extent practical.

  - Process: Design and Development Task 16 – Determine if the design was correctly transferred to production.

  - Process: Production and Service Controls Task 4 – Determine if the organization has established documented requirements for product cleanliness including any cleaning prior to sterilization, cleanliness requirements if provided non-sterile, and assuring that process agents are removed from the product if required.

  - Process:

  - Process: Production and Service Controls Task 16 – Determine if the manufacturer has established and maintained a file for each type of device that includes or refers to the location of device specifications, production process specifications, quality assurance procedures, traceability requirements, and packaging, labeling specifications, and when applicable requirements for installation and servicing. Confirm that the manufacturer determined the extent of traceability based on the risk posed by the device in the event the device does not meet specified requirements.

- PROCEDURE

  - Contents of the Medical Device File

    - A Medical Device File (MDF) is developed and maintained for all Tulip Medical Device Product Families.

    - Complete Form 4.2-5 Medical Device File to include, or make references to:

- General description

- Product specifications

- Specifications and procedures

- Manufacturing procedures (include all of the procedures and work instructions for production, product verification, process validation, identification, inspection status, etc.)

- Packaging procedures

- Storage procedures

- Handling procedures

- Distribution procedures

- Measuring and Monitoring

- Product verification activities including the Safety and efficacy of the device.

- Process control SPC

- Installation requirements

- Servicing procedures

  - Establish records in accordance with the requirements of the MDR

    - The MDR 2017/745 requires a quality management system that ensures products conform to all applicable regulatory requirements. The technical documentation, as outlined in MDR Annex II and supported by ISO 13485:2016, must demonstrate conformity with the applicable General Safety and Performance Requirements (GSPRs) of Annex I. This includes maintaining records and evidence that the device meets the essential criteria for safety, performance, and regulatory compliance throughout its lifecycle.

  - Medical Device File Record Locations

    - The MDF must be maintained by the Management Representative (MR) in a manner to facilitate retrieval.

  - Record Keeping Requirements

    - Each MDF will be prepared, dated and approved by the MR. Maintain current information on Form 4.2-5 Medical Device File.

- FLOWCHART

  - N/A