Tulip Medical

Quality System Procedure

QSP 5.4-1

Quality Planning

Approvals

VP Regulatory Affairs	Date

Regulatory Affairs Coordinator	Date

Document Control Administrator	Date

| Rev | Date | Description of Change |
| --- | --- | --- |
| 0 | 08/07/2013 | Released documentation – Implementation of document for compliance to FDA QSR Subpart B 820.20 Quality System and ISO 13485 5.4 Quality Planning. |
| 1 | 09/18/2014 | Updated to ISO 13485:2012 |
| 2 | 09/26/2018 | Reformatted document and added new signature block. |
| 3 | 5/23/2025 | 3.0 Added MDR Regulation 7.5.3 Updated from Drop box to Change request log 7.11Updated mentions from MDD to MDR Removed Appendix A and B |

- PURPOSE

To describe how Tulip Medical controls required Quality System documents and our website. These controls ensure personnel have access to the latest revisions of controlled documented they need to do their jobs. Such controlled documents include, but may not be limited to, procedures, forms, the Quality Manual, labeling, information on the website, product drawings and specifications, and external documents such as ISO 13485:2016.

This procedure assigns responsibilities for establishment, review, authorization, issue, distribution, and revision of controlled documents.

Note: All Quality System documentation is considered confidential and must not be delivered outside the company without prior approval of the Management Representative (MR) or President.

- SCOPE

This Quality System Procedure is to be followed by all Tulip Medical Products personnel. Comprehension of, and satisfactory expertise in, the material covered by this Quality System Procedure by each employee will be indicated through the completion of all applicable and appropriate training requirements, as is indicated by the completion of Form 6.2-1-2 Training Record in accordance with QSP 6.2-1 Training and Qualification.

- REFERENCE DOCUMENTS

| Regulation (EU) 2017/745 on medical devices (MDR) |  |
| --- | --- |
| Food and Drug Administration 21 CFR 820 Quality System Regulation |  |
| International Standard Medical Devices – Quality Management Systems – Requirements for Regulatory Purposes ISO 13485:2016 |  |
| Form 4.2-1-L | Change Request Log |
| Form 4.2-1-2 | Change Request |
| Form 6.2-1-2 | Training Record |
| Form 7.1-1-1 | Project Control Form |
| QSP 4.2-2 | Control of Quality Records |
| QSP 5.6-1 | Management Review |
| QSP 6.2-1 | Training and Qualification |
| QSP 7.1-1 | Risk Management |
| QSP 7.3-2 | Device Master Record |
| QSP 8.2-1 | Quality Audits |
| QSP 9.1-3 | Technical Documentation - MDR |

- DEFINITIONS

Controlled Documents – Quality System documentation, including the Quality Manual, QSPs, Work Instructions, forms, product specifications, product drawings, Device Master Records, product labeling, downloadable instructions, and external standards and regulations.

A document is defined as controlled and considered current if, and only if, it is located in the ‘Controlled Documents’ section of the company’s computer server. There is one, and only one, copy of each controlled document.

Document Control – Consists of Regulatory Affairs (RA) and Quality Assurance (QA).

Documentation System Structure – Tier 1: Quality Manual / Quality Policy

Tier 2: Quality System Procedures

Tier 3: Work Instructions, or specific department instructions’ product-specific documents, including drawings; and other Device Master Record documents.

Tier 4: Forms

External Documents – Documents that originated outside of the company (e.g. standards such as ISO 13485:2016, FDA regulations, etc.) and over which Tulip Medical does not have change control authority.

Internal Documents – Documents that are created by Tulip Medical and/or which the organization changes and approves via this document control system.

Management Representative – The member of Tulip Medical whose cross-functional role encompasses the development and monitoring of the quality management system to ensure accuracy and effectiveness of all Quality System Procedures (QSPs).

Records – Documented evidence, either hard copy or electronic media, of a particular event or events, as related to the operation of the Quality Management System. Records include completed forms.

- ABBREVIATIONS

DC – Document Control

DCL – Document Control List

EVP – Executive Vice President

MR – Management Representative

QA – Quality Assurance

QSP – Quality System Procedure

RA – Regulatory Affairs

- RESPONSIBILITIES

The Management Representative is responsible for maintaining this procedure in a current state, and for the implementation and management of controlled documents.

All employees are responsible for controlling and handling documentation in accordance with this procedure’s requirements.

The originator and reviewers of documents are responsible for assuring the documents are clear, concise, and accurately describe the process being documented.

- PROCEDURE

This procedure is to be followed when generating a new document or revising an existing controlled document to describe a process, or work, being accomplished at the company. This may arise from:

- Any Quality System Procedures or forms, including Work Instructions.

- Product specifications, Device Master Record documents, Technical Files, etc.

- Any labels and Instructions For Use for Tulip Medical Products. This includes items on the company’s website.

- Any other document required for the Quality Management System.

  - Document Numbering

    - All new documents and procedures are assigned a unique document name (title) and starting revision level and date. Procedures and forms are each given a unique document number and revision level. This number will be recorded in the applicable section of the Document Control List (DCL).

    - Revision Levels – New controlled procedures and forms receive a revision level of 0 (zero). As the document is revised, it is given the next sequential revision number (i.e. Rev. 1, 2, 3, etc.).

    - If a document is being modified, update the document and give it a new revision level. However, if the document is being replaced with a new document, obsolete the old document and give the new document a new document number.

    - The document becomes effective on the date that all the required signatures are in place, Form 4.2-1-2 Change Request is completed, and the document is placed in the ‘Controlled Documents’ section of the company’s server, or on the website.

  - Controlled Documents are reviewed for the following:

    - Content and suitability, including clear directions.

    - Compliance with applicable regulatory documents.

    - Associated procedures, work instructions, forms, or records.

  - Minimal Approval Signatures

    - The Quality Manual requires the approval signature of the CEO. QSPs will be approved by the Executive Vice President (EVP) or MR.

    - The Quality Policy is controlled through the Quality Manual and as such does not require any separate signature(s) to show its approval. If a revision to the Quality Manual revises the Quality Policy, that revision of the Quality Manual must be approved by the CEO.

    - The Quality Objectives are approved through the approval of the Management Review minutes (see QSP 5.6-1 Management Review). The Management Review minutes require the approval signature of the CEO.

    - Work Instructions require one of the following approval signatures: The EVP or Operations.

    - Tulip Medical’s Device Master Record product-specific documents each require the approval signature of the CEO or MR (see QSP 7.3-2 Device Master Record). Additionally, changes to product-specific documents that make up the Device Master Records also require Form 4.2-1-2 Change Request to be filled out (see Document Control of Product-Specific Documents, below).

    - Technical Files, Clinical Evaluations and Risk Management / Risk Assessments / Risk Management Plans require the review and approval of the CEO and/or the MR. Declarations of Conformity require the review and approval of the EVP and/or the MR

Note: The designated functions have access to pertinent background information upon which to base their approval.

  - Document Control of Non-Product-Specific Procedures and Forms – The Quality Manual, QSPs, Work Instructions, and forms.

    - Non-product-specific procedures and forms are reviewed and approved by the authority, or authorities, on the cover page of the document as indicated above. The signature(s) indicate the person signing the document has reviewed and approved the document.

    - The electronic copies of documents in the ‘Controlled Documents’ section of the company’s server are considered controlled and current.

    - Procedures include details of the changes to the document at each revision.

    - Control of Forms

      - Only the content of forms is controlled (i.e., the format, font, spacing, etc. is not controlled and may be changed without officially revising the form).

      - Forms related to QSPs or Work Instructions are considered reviewed, approved and controlled by the approval signature of the QSP or Work Instruction they correspond to, as well as by their placement in the ‘Controlled Documents’ section of the company server, maintained by DC. For example, the review and approval of this procedure, QSP 4.2-1 Document Control, indicates the review and approval of Form 4.2-1 Change Request and any other forms directly corresponding to this QSP.

    - For information on back-up of electronic document, see QSP 4.2-2 Control of Quality Records.

  - Document Control of Product-Specific Documents That Form the Device Master Records.

    - Tulip Medical maintains Device Master Records as described in QSP 7.3-2 Device Master Record.

    - For changes to product-specific documents that make up the Device Master Records that could potentially involve risk, complete Form 4.2-1-2 Change Request. Form 4.2-1-2 Change Request requires the approval signature of the COO or MR.

Note: An example of a change to a product-specific document that does NOT potentially involve risk would be a change to a logo. The change of a logo might change the product-specific documents, but would NOT require a Change Request since there is no realistic possibility of death, injury, or damage from a new logo.

    - The Change Request Log will be used to keep track of Change Requests. Each Change Request form will be given a sequential number in the format XX-YYY. XX will be the current year and YYY will be the next sequential number.

    - Any number of product-specific documents may be revised through a single Change Request form, as long as all the revised documents pertain to the same product specification change. All documents affected will be listed on Form 4.2-1-2 Change Request.

    - Device Master Record documents are considered controlled and current by their placement in the appropriate Device Master Record, which in turn is maintained in the ‘Controlled Documents’ section of the company’s server.

  - Document Control of Technical Files, Declarations of Conformity, Risk Management documents, and Clinical Evaluations.

    - Technical Files, Declarations of Conformity (reference QSP 9.1-3 Development of Technical File), Risk Management documents (reference QSP 7.1-1 Risk Management) and Clinical Evaluations are approved by Tulip Medical. They are considered controlled by their placement in the ‘Controlled Documents’ (Technical File) maintained by DC in the ‘Controlled Documents’ section of the company’s server.

  - Product Information / Labeling on the Company’s Website.

    - Product information and product labeling changes on the company’s website will be reviewed, approved, and controlled by the MR / COO. Copies of e-mails to the web controller will be retained as quality evidence. All information that describes the products or intended use will be controlled by taking a screenshot of the new information on the website, which will be approved by the MR. A copy of this approval will be filed by DC. All downloadable IFUs and Instructions will have a unique name to include a revision number. These items will be tracked on the DCL. Any changes to website labeling items will be controlled by the use of Form 4.2-1-2 Change Request.

  - Changes to Tulip Medical Products drawings will be controlled by the use of Form 4.2-1-2 Change Request. Operations will be responsible for sending new drawings to vendors and updating the inventory control software for that item with the new revision number for all purchase orders.

  - For prototypes that are being put into production after testing, Form 4.2-1-2 Change Request will be completed in addition to the information contained on Form 7.1-1-1 Project Control Form. Complex changes, or the introduction of a new product line, will follow the requirements of QSP 7.3-1 Design and Development.

  - Revision History

    - Procedures will contain or reference a history of the changes that have been made to the various revisions. It is only necessary to maintain the revision history for the last five document revisions.

  - External Documents

    - External documents that are relevant to Tulip Medical’s Quality Management System, such as ISO 13485 / ISO 14969, are controlled by their placement in the ‘Controlled Documents’ section of the company’s server maintained by the MR / DC. Form 4.2-1-2 Change Request will be utilized to document how the change affects Tulip Medical Products, as well as to determine any required actions.

      - Additionally, national regulatory requirements for the various countries we sell to (including FDA Quality System Regulations, Canadian Medical Device Regulations SOR/98-282, and Regulation (EU) 2017/745 on medical devices (MDR) as well as European MEDDEVs) can be found online at:

        - http://www.global.ihs.com

        - http://ed.europa.eu/health/medical-devices/documents/guidelines/index_en.htm

      - Emergo Group sends out e-mails notifying of changes to many external regulatory documents. In addition, the following websites may be used to review the status of controlled external documents:

        - http://www.newapproach.org/Directives/Default.asp

        - http://www.accessdata.fda.gov/scripts/cdrh/cfdoc/cfStandards/search.cfm

        - http://www.hc-sc.gc.ca/dhp-mps/md-im/index-eng.php

      - Tulip Medical Products will periodically review the EU MDR/ISO, Health Canada, and FDA websites to check for changes and updates.

      - Annually, prior to the Management Review, the external documents will be reviewed against the current ISO documents to verify that Tulip Medical Products has an up to date list of external documents.

  - Training as Follow Up to Document Control Changes

    - The MR / DC notifies appropriate personnel that documents have been added or changed so that training can be conducted as required for the particular document (see QSP 6.2-1 Training and Qualification). Training to additions or changes occurs as soon as possible after the latest revision of the document is controlled to ensure that work is not performed to an obsolete document.

Note: In the event that training pertains to a document or process that directly affects a process followed by Production Personnel in the handling of the components of the items sold by Tulip Medical Products, training is to be performed prior to the formal implementation of a revised document into the system through placement in the ‘Controlled Documents’ section and removal of the obsolete document, per the requirements of QSP 6.2-1 Training and Qualification.

  - Document Review During Use

    - Company personnel are encouraged to critically evaluate the documents they use and request changes to processes, as well as correct errors and inconsistencies.

    - Quality audits are performed on the Quality Management System processes. The results of audits may lead to updates to controlled documents (see QSP 8.2-1 Quality Audits).

  - Obsolete Documents

    - When a controlled document has been superseded by a new revision, or the document has been otherwise removed from the Quality Management System, segregate the document from current documents in a clearly identified location. If the document has been revised, replace the superseded obsolete document with the updated revision.

    - Obsolete copies of Tulip Medical, Inc. controlled documents are filed by the MR / DC and retained according to QSP 4.2-2 Control of Quality Records. There must be at least one (1) copy of the obsolete document maintained.

    - Regulatory notifications related to significant organizational or product changes are addressed in QSP 9.1-3 Technical Documentation – MDR and associated regulatory communication procedures.