Tulip Medical

Quality System Procedure

QSP 7.4-2 Product Inspection

Approvals

V.P Regulatory Affairs 						             Date

Regulatory Affairs Coordinator					             Date

Document Control Administrator					              Date

| Rev | Date | Description of Change |
| --- | --- | --- |
| 0 |  | Initial release. |
| 1 | 04/15/2013 | 04/15/13 Added ISO 2859-1 Inspection Table. Removed Master Document List Reference. |
| 2 | 05/07/2013 | 05/07/13 Added Form 7.4-2-1 Certificate of Conformity |
| 3 |  | Updated Table 1 per ISO 2859-1 |
| 4 |  | Reformatted document. Section for ‘Scope’ added, and updates made to ‘References’ and ‘Definitions.’ Procedure updated to reflect current procedure used. |
| 5 | 10/02/2019 | Section 2.0 Scope – updated paragraph Section 3.0 Reference – added QSP 7.2-1 Sales Orders Section 4.0 Definitions, Finished Components, and In-Process Goods – updated definitions Section 6.0 Responsibilities – changed from All Production Personnel to ‘Production Manager’ Section 7.0 Procedures – changed all finished ‘goods’ to finished ‘products’; 7.1.1, 7.1.2 updated paragraphs; Section 7.2 Perform initial inspection – 7.2.1, added location; 7.2.4, updated paragraph Section 7.3 Performing receiving inspection – 7.3.4, 7.3.5 – update paragraphs Section 7.5 If Components are Accepted – updated title; 7.5.1, 7.5.2, 7.5.3 updated paragraphs; added 7.5.4. Section 7.6 Package Components, Perform Packaging Inspection and Final Product Inspection – 7.6.1, 7.6.2, 7.6.2, Section 7.7 Final Product Inspection; 7.7.1 update paragraph. Section 8.0 Flow Chart – corrected according to procedures |
| 6 | 05/12/2025 | 3.0 Updated Reference Documents to align with QSP 6.0 Added Production Personnel 7.0 Simplified process to improve efficiency by cutting out repeated steps without sounding like a detailed work instruction. 8.0 Updated flowchart |

- PURPOSE

To document the procedural controls that Tulip Medical products has developed to verify and inspect incoming goods, in-process and finished product at Tulip Medical Products.

- SCOPE

This procedure applies to all sub-part, parts and product inspections performed by Tulip Medical Products.

- REFERENCE DOCUMENTS

| Form 1-1 | Receiving Inspection Damage Documentation Record |
| --- | --- |
| Form 6.2-1-2 | Training Record |
| Form 7.4-2-1 | Receiving Inspection Form |
| Form 7.5-1-2 | Final Inspection – Reusable Cannulas |
| Form 7.5-1-3 | Single-Use Assembly for Accessories |
| Form 7.5-1-4 | Single-Use Assembly for Cannulas |
| Form 7.5-3 | Packaging: Kits & Sets – All Single-Use Accessories, Kits, Cannulas |
| Form 7.5-8 | Product Marking & inspection – Engraved Reusable Accessories |
| Form 8.3-1-1 | Nonconformance Report |
| QSP 6.2-1 | Training and Qualification |
| QSP 8.3-1 | Control of Nonconforming Product |
| RFD-009 | Product Inspection Sample Size Chart |
| WI 1 | Product Inspection – Reusable |
| WI 1.2-1 | Single-Use Hubs |
| WI 1.3 | Product Inspection – Pro-Tech Sterile/Packaged Single-Use |

- DEFINITIONS

Finished Components – Components that are completed from a manufacturing standpoint.  These components have not been boxed in sets or kits.

In-Process Goods – Components and/or materials that are partially completed and are currently in the manufacturing or boxing in sets or kits process.

Management Representative – The member of Tulip Medical whose cross-functional role encompasses the development and monitoring of the quality management system to ensure accuracy and effectiveness of all Quality System Procedures (QSPs).

Raw Materials – Materials that have not yet been processed or manufactured into the final form.

- ABBREVIATIONS

MR – Management Representative

PO – Purchase Order

QA – Quality Assurance

- RESPONSIBILITIES

The Management Representative is responsible for maintaining this procedure in a current state, and for the implementation and management of controlled documents.

Production Manager is responsible for controlling and handling documentation in accordance with this procedure’s requirements.

Production Personnel must be trained and documented in accordance with QSP 6.2-1 Training and Qualification procedures to perform final inspections on products and/or build assemblies

- PROCEDURE

  - Receiving raw materials, in-process, and finished components at Tulip Medical Products

    - Components received at Tulip Medical may include reusable or single-use raw materials or finished components used to box into sets or kits prior to fulfilling customer orders.

    - All components are held in the Receiving Area until the QA inspection is performed and completed. Only then are components moved to their applicable location.

    - Form 7.4-2-1 Receiving Inspection is to be completed for all incoming raw material, in-process, and finished components

  - Perform Initial Receiving Inspection

    - When raw materials, in-process, and/or finished components are received, retrieve the Tulip Medical Product Purchase Order (PO) for the shipment.  POs are in one of two locations:

    - Traveler in Receiving Area

    - SOS company software under Purchase Order tab

    - Check the Packing Slip to ensure that the shipment is for Tulip Medical Products.

    - Sign, date, and initial each Packing Slip.

    - Visually inspect the following:

- Outside of the received box/bag - any damages, opened when received, punctures, etc. complete Form 1-1 Receiving Inspection Damage Documentation Record

- Inside of the received box - verify that the packaging and goods inside are not damaged.

    - Ensure all corresponding documents are provided material certifications and/or Certificates of Compliance (if applicable).

    - Perform inspection in accordance with the procedures outlined in work instructions

- WI 1 Product Inspection – Reusable

- WI 1.2-1 Single-Use Hubs

- WI 1.3 Product Inspection – Pro-Tech Sterile/Packaged Single-Use

    - The Production Department documents receipt of all items received on Form 7.4-2-1 Receiving Inspection Form.

    - Upon release of incoming components label all containers and/or bags with the appropriate Approved Sticker, which signifies the components are approved for production use and distribution.

    - Accepted components/product, are to be moved to their allocated area

    - Rejected component/product, if any nonconformance are found separate and proceed with Form 8.3-1-1 Nonconformance Report

  - Product Inspection

    - All cannulas, accessories, and kits produced by Tulip are subject to 100% inspection during final inspection. Based on the product type, one of the applicable forms listed below must be completed in accordance with its corresponding Work Order and Work Instruction

      - Ready To Sell Products

- Form 7.5-1-2 Final Inspection – Reusable Cannulas

- Form 7.5-3 Packaging: Kits & Sets – All Single-Use Accessories, Kits, Cannulas

- Form 7.5-8 Product Marking & inspection – Engraved Reusable Accessories

      - Assembly Builds for Sterilization

- Form 7.5-1-3 Single-Use Assembly for Accessories

- Form 7.5-1-4 Single-Use Assembly for Cannulas

    - Rejected products are to proceed with QSP 8.3-1 Control of Nonconforming Product.

- PROCESS FLOWCHART

Complete in accordance with QSP 8.3-1

7.1 Receiving at Tulip

7.2 Perform Initial Receiving Inspection with Form 7.4-2-1

Accept / Reject

Rejected, initiate

Form 8.3-1-1

Products ready for sale

7.3.1.2 Assembly build ready for sterilization

Accept / Reject

7.3 Product Inspection with corresponding form

7.2.9 Accepted, move to allocated area

Components to be used

in assembly