Tulip Medical

Quality System Procedure

QSP 7.3-2

Device Master Record

Approvals

VP Regulatory Affairs	Date

Regulatory Affairs Coordinator	Date

Document Control Administrator	Date

| Rev | Date | Description of Change |
| --- | --- | --- |
| 0 | 05/06/2013 | Initial release. |
| 1 | 07/16/2014 | Update 7.1 for sterilization requirements. |
| 2 | 09/11/2018 | Reformatted document. |
| 3 | 5/23/2025 | Revised the purpose to align with ISO 13485:2016 and Regulation (EU) 2017/745 (MDR). Updated the scope to reflect that the Device Master Record (DMR) is a regulatory requirement under MDR instead of FDA. Added MDR and ISO 13485:2016 references to the Reference Documents section. Reviewed and updated terminology throughout the document to ensure alignment with MDR requirements, particularly with respect to Technical Documentation as outlined in MDR Annex II and III. |

- PURPOSE

The purpose of this procedure is to define the requirements for preparing, maintaining, and controlling Device Master Records (DMRs) for all medical devices manufactured by Tulip Medical Products in accordance with the requirements of ISO 13485:2016 and Regulation (EU) 2017/745 (MDR).

- SCOPE

A Device Master Record is required for all devices manufactured by our own company. This is a regulatory file required by Regulation (EU) 2017/745 (MDR). Device Master Records may be reviewed by our Notified Body, Health Canada, FDA, and the European Competent Authorities, as appropriate.

This procedure is to be followed by all Tulip Medical Products personnel. Comprehension of, and satisfactory expertise in, the material covered by this Quality System Procedure by each employee will be indicated through the completion of all applicable and appropriate training requirements, as is indicated by the completion of Form 6.2-1-2 Training Record in accordance with QSP 6.2-1 Training and Qualification.

- REFERENCE DOCUMENTS

| Regulation (EU) 2017/745 on Medical Devices (MDR) |  |
| --- | --- |
| International Standard Medical Devices – Quality Management Systems – ISO 13485:2016 |  |
| Form 6.2-1-2 | Training Record |
| Form 7.3-2-1 | Device Master Record Index |
| QSP 4.2-1 | Document Control |
| QSP 6.2-1 | Training and Qualification |
| QSP 8.2-3 | Returns & Exchanges |

- DEFINITIONS

Device Master Record – From FDA’s Quality System Regulations, section 820.3 (j); A compilation of records containing the procedures and specifications for a finished device.

Under MDR 2017/745, the Device Master Record (DMR) supports the compilation of Technical Documentation required by Annex II and III

Management Representative – The member of Tulip Medical whose cross-functional role encompasses the development and monitoring of the quality management system to ensure accuracy and effectiveness of all Quality System Procedures (QSPs).

- ABBREVIATIONS

DMR – Device Master Record

FDA – Food and Drug Administration

MR – Management Representative

- RESPONSIBILITIES

The Management Representative (MR) is responsible for maintaining this procedure in a current state, as well as maintaining the control of the contents of each product’s Device Master Record.

- PROCEDURE

  - Contents of Device Master Records (DMRs)

    - A DMR is developed and maintained for all Tulip Medical Products medical devices or product families.

    - The information includes, or makes reference to:

- Device specifications, including appropriate drawings, composition, formulation, component specifications and software specifications (if applicable).

- For product manufactured by Tulip Medical Products – Production process specifications, including the appropriate equipment specifications, production procedures and production environment specifications. For product manufactured by an approved supplier, the supplier maintains such records.

- Quality assurance procedures and specifications, including acceptance criteria and the quality assurance equipment to be used.

- Sterilization requirements to include sterility assurance level, method of sterilization, sterilization cycle conditions and aeration conditions.

- Packaging and labeling specifications, including processes used.

- Installation is not currently applicable to Tulip Medical Products. Maintenance of the devices, if applicable, is described in the device Instructions for Use. For servicing procedures and methods, reference QSP 8.2-3 Returns & Exchanges.

  - DMR Locations

    - The information included or referenced in the DMR must be maintained in a manner to facilitate retrieval.

    - The DMR will be maintained by the MR.

  - Record Keeping Requirements

    - Each DMR will be prepared, dated and approved by the MR. The contents of each DMR are recorded on Form 7.3-2-1 Device Master Record Index for that product.

    - Any changes to the DMR will be documented according to QSP 4.2-1 Document Control.

- FLOWCHART

  - N/A