Tulip Medical

Quality System Procedure

QSP 7.1-1 Project System Control

Approvals

VP Regulatory Affairs	Date

Regulatory Affairs Coordinator	Date

Document Control Administrator	Date

| Rev | Date | Description of Change |
| --- | --- | --- |
| 0 | 05/06/2013 | Initial release. |
| 1 | 05/08/2015 | Complete rewrite to define controls for projects and prototypes. |
| 2 | 08/28/2018 | Reformatted. Addition of Section 2.0 scope. Sections 3.0, 4.0, 5.0, 6.0 and 7.0 updated for accuracy in accordance with ISO 13485:2016. |
| 3 | 5/23/2025 | Reviewed for MDR 2017/745 compliance Added MDR ,ISO and forms to cross reference with the QSP |

- PURPOSE

The purpose of this procedure is to define the necessary controls and processes for managing projects at Tulip Medical Products in accordance with Regulation (EU) 2017/745 on medical devices (MDR) and the principles of risk management as outlined in EN ISO 14971:2019.

- SCOPE

This Quality System Procedure is to be followed by all Tulip Medical Products personnel. Comprehension of, and satisfactory expertise in, the material covered by this Quality System Procedure by each employee will be indicated through the completion of all applicable and appropriate training requirements, as is indicated by the completion of Form 6.2-1-2 Training Record in accordance with QSP 6.2-1 Training and Qualification.

- REFERENCE DOCUMENTS

| Regulation (EU) 2017/745 on Medical Devices (MDR) |  |
| --- | --- |
| EN ISO 14971:2019 Medical devices – Application of risk management to medical devices |  |
| Form 4.2-1-2 | Change Request |
| Form 6.2-1-2 | Training Record |
| Form 7.1-1-1 | Project Control Form |
| QSP 6.2-1 | Training and Qualification |
| QSP 7.3-1 | Design and Development |

- DEFINITIONS

Management Representative – The members of Tulip Medical whose cross-functional role encompasses the development and monitoring of the quality management system to ensure accuracy and effectiveness of all Quality System Procedures (QSPs).

- ABBREVIATIONS

MR – Management Representative

PM – Production Manager

QA – Quality Assurance

- RESPONSIBILITIES

Quality Assurance (QA) is responsible for maintain this procedure in a current state.

All employees are responsible to ensure that efforts directed towards producing the medical device products comply with the requirements of the Quality System.

- PROCEDURE

  - Planning of Product Realization

    - Production staff, MR, PM, QA and EVP coordinate plans, and develop and manage the processes needed for product realization during scheduled meetings. Planning of product realization is consistent with the requirements of the other processes of the quality management system.

    - During these product realization (gateway) meetings, include one or more processes for risk management in product realization.

      - Product realization, these gateway meetings will determine the following, as appropriate.

        - Quality objectives and requirements for the product;

        - The need to establish processes and documents (see 4.2.4) an to provide resources specific to the product, including infrastructure and work environment;

        - Required verification, validation, monitoring, measurement, inspection and testing, handling, storage, distribution and traceability activities specific to the product together with the criteria for product acceptance;

        - Records needed to provide evidence that the realization processes and resulting product meet requirements (see 4.2.5).

      - The output of this planning shall be documented in a form suitable for the organization’s method of operations.

Note: Further information can be found in ISO 14971.

      - Records of risk management activities shall be maintained.

  - All requests for custom made orders, prototypes and special use items will be forwarded to the Management Representative (MR) for review and disposition. For all large or complex requests, a Quality Plan and cost estimate will be completed for a business case analysis prior to proceeding. For all major deviations from existing products, new product design or at the Production Manager’s (PM) discretion, the design control requirements from QSP 7.3-1 Design and Development will be invoked.

  - Form 7.1-1-1 Project Control Form will be filled out for all approved projects and logged by the MR into the excel spreadsheet maintained in Dropbox for Project Control. This request will go to the PM for manufacturing or sourcing. A separate item will be manufactured for Tulip Medical Products to retain for follow-up testing if required.

  - The PM will be responsible for determining and executing the required testing based on the project complexity and intended use.

  - Once the project is completed, one item will be provided to the MR. The other will be sent to the person requesting the item. Form 7.1-1-1 Project Control Form will be completed.

  - For all projects which will be put into production, Form 7.1-1-1 Project Control Form and Form 4.2-1-2 Change Request will be completed to document process/product validation testing and determine actions based on the change to the product line.

  - The PM and MR will ensure that the Design History Files and/or Device Master Records is/are updated as required for all items put into production. The Document Control Log will be updated for the new drawing.

- PROCESS FLOWCHART

  - N/A