Tulip Medical

Quality System Procedure

QSP 7.5-1

Device History Record

Approvals

VP Regulatory Affairs	Date

Regulatory Affairs Coordinator	Date

Document Control Administrator	Date

| Rev | Date | Description of Change |
| --- | --- | --- |
| 2 |  | 5/7/2013 Single Eagle Labs and their Subcontracted Sterilization Company Isomedics supply a critical sterilization service, a Quality Management System on-site audit of Eagle Labs and any sterilization run records will be required at least every 4 years. A record of the audit will be maintained in the quality system, under Audits, Subfolder, Supplier Audits, Eagle Labs. |
| 3 |  | 05/20/2013 Step 6 on requirements for supplier audits was removed. |
| 4 |  | Update required documentation to be maintained in the Device History Records. |
| 5 |  | 06/23/2017 Change mentions of Netsuite Accounting software to SOS Inventory. Change management representative to Marc Pilkington. |
| 6 |  | Updated for accuracy. |
| 7 | 10/01/2019 | Added the ‘Date’ column to this page Section 5.0 Abbreviation – added DCA – Document Control Administrator Section 6.0 Responsibilities – added DCA responsibilities Section 7.3 Record Keeping Requirements – 7.3.1 removed MR as maintaining record; 7.3.2 added DCA who maintains the documents and logs |
| 8 | 5/23/2025 | Added reference to EU MDR 2017/745 in the Purpose section to support compliance with Annex XI (Part A) and Annex IX (Chapter I, Section 2.2) regarding production quality assurance and DHR requirements for EU regulatory audits. |

- PURPOSE

The purpose of this procedure is to outline the method for establishing and maintaining Device History Records. The Device History Records are maintained according to written procedures to demonstrate that the finished device is manufactured in accordance with the Device Master Record.

- SCOPE

This procedure applies to all devices under the control of Tulip Medical.

- REFERENCES

This procedure also supports compliance with EU Regulation 2017/745 (MDR), Annex XI, Part A (Production Quality Assurance), and Annex IX, Chapter I, Section 2.2, which require the manufacturer to ensure each device batch is manufactured in accordance with the product documentation and subjected to appropriate release criteria.

- DEFINITIONS

Device History Record – A compilation of records containing the complete production history of a finished device, as according to CFR 820.184.

Management Representative – The member of Tulip Medical whose cross-functional role encompasses the development and monitoring of the quality management system to ensure accuracy and effectiveness of all Quality System Procedures (QSPs).

- ABBREVIATIONS

DCA – Document Control Administrator

DHR – Device History Record

MR – Management Representative

- RESPONSIBILITIES

The Management Representative (MR) is responsible for maintaining this procedure in a current state, and for ensuring that Tulip Medical employees whose jobs encompass these procedures follow them accordingly.

Operations is responsible for maintaining the control of the contents of each product’s Device History Record (DHR).

Document Control Administrator is responsible for maintaining the most recent revision of this document in the manual Master Files.

- PROCEDURE

  - Preparation of DHR

    - The DHR is prepared for each lot of devices manufactured by Tulip Medical Products, or any authorized second or 3rd party manufacturer on behalf of Tulip Medical Products.

    - Each DHR contains production and quality control records that are traceable by unique lot numbers and/or serial numbers. These numbers can be created from Sales Order numbers, supplier Assembly Numbers and/or they can be assigned by Operations. Lot numbers are referenced in the SOS Inventory software and are traceable to suppliers and dates of manufacture.

  - DHR Required for Each Manufactured Product and Content

    - Each DHR includes or references

      - Product name

      - The Date(s) of Manufacturer

      - The Quantity Manufactured

      - The Quantity Released

      - Applicable Lot Number(s)

      - Applicable Serial Number(s) – “Unique Identifiers,” if required

      - Product Release records to include receiving and final inspection forms for product and services used during the production process

      - The primary labels used for the production unit, and the box if it is sold in packs

      - Sterilization run records and certificates of sterility/conformity from a 3rd third party Sterilization Company

      - The QA release signature on Final Inspection Forms certifies that the applicable records have been checked and the product is released as finished goods according to it’s appropriate DMR.

  - Record Keeping Requirements

    - All Tulip Medical Products DHRs contents are maintained by Operations.

    - After manufacturing records and all pertinent labeling and test/inspection data have been completed the files are maintained and revisions are logged by the DCA.

    - DHRs are filed by applicable product.

- FLOWCHART

  - N/A