Tulip Medical

Quality System Procedure

QSP 8.4-1

Testing, Measurement and Analysis

Approvals

V.P. Regulatory Affairs 	Date

Regulatory Affairs Coordinator	 Date

Document Control Administrator 	 Date

| Rev | Date | Description of Change |
| --- | --- | --- |
| 0 | 05/06/2013 | Initial release. |
| 1 | 05/15/2014 | Add test protocol and reporting guidance. |
| 2 | 08/29/2018 | Reformatted documented. |
| 3 | 09/25/2019 | Added guidelines for sampling plan |
| 4 | 3/13/2024 | Added section 7.8.7 Requirements for Analysis Data |
| 5 | 3/13/2025 | 2.0 Scope expanded to include Post Design processes and correlations to related procedures. 3.0 Updated reference documents List 4.0 Added to definitions 5.0 Corrected abbreviation for CAPA 7.4 Added data collecting summary for QSPs mentioned on the additional scope 7.7.1 Rephrased for clarity 8.0 Updated Flowchart |
| 6 | 5/23/2025 | Added MDR regulatory reference to Scope section to reflect alignment with Regulation (EU) 2017/745. Updated Reference Documents list to include MDCG 2023-3 and MDCG 2023-4 to support trending and PMS data analysis guidance. Clarified terminology to reflect MDR-compliant language where applicable. |

- PURPOSE

The purpose of this procedure is to document how information on our products and processes is gathered, analyzed and actions taken so that we comply with the planned arrangements of our Quality Management System. Additionally, this QSP provides guidance for Protocol and Test Reports to be used for new and future product testing. Testing, measurement and analysis are instrumental for product improvement, the development of new products, and the Quality Management System.

- SCOPE

This Quality System Procedure is to be followed by all Tulip Medical Products personnel. Comprehension of, and satisfactory expertise in, the material covered by this Quality System Procedure by each employee will be indicated through the completion of all applicable and appropriate training requirements, as is indicated by the completion of Form 6.2-1-2 Training Record following QSP 6.2-1 Training and Qualification.

This procedure applies to production, post-production processes, and all stages of the quality management system where statistical techniques are required, including integration with QSP 7.4-2, QSP 7.4-5, QSP 9.1-7, QSP 5.6-2, and QSP 7.3-3."

This procedure supports compliance with Regulation (EU) 2017/745 on medical devices, particularly Annex III (Technical Documentation on Post-market Surveillance), and Annex I, Chapter I (General Safety and Performance Requirements).

- REFERENCE DOCUMENTS

Regulation (EU) 2017/745 on Medical Devices (MDR)

| Form 6.2-1-2 | Training Record |
| --- | --- |
| QSP 5.6-1 | Management Review |
| QSP 5.6-2 | QMS Risk Management Plan |
| QSP 6.2-1 | Training and Qualification |
| QSP 7.3-3 | Risk Management |
| QSP 7.4-2 | Product Inspection |
| QSP 7.4-5 | Process Validation |
| QSP 8.2-2 | Customer Feedback / Complaints |
| QSP 8.3-1 QSP 8.4-3 | Control of Nonconforming Product Statistical Sampling Plan for Verification & Validation |
| QSP 8.5-1 | Corrective and Preventive Action |
| QSP 9.1-7 | Post Market Surveillance |
| RFD-009 | Product Inspection Sample Size Chart |
| MDCG 2023-3 | Guidance on Vigilance – trending of events and complaints |
| MDCG 2023-4 | PMS module guidance including statistical analysis and feedback |

- DEFINITIONS

COMPLAINT - Any written, electronic, or oral communication that alleges deficiencies related to the identity, quality, durability, reliability, safety, effectiveness, or performance of a device after it is released for distribution

FEEDBACK: Opinions, comments and expressions of interest in a product, a service, or a complaints-handling process

NONCONFORMITY - The nonfulfillment of a specified requirement

POST-MARKET SURVEILLANCE (PMS): All activities carried out by manufacturers in cooperation with other economic operators to proactively collect and review experience gained from devices they place on the market, in order to identify any need to immediately apply any necessary corrective or preventive actions.

TREND REPORTING: The identification of statistically significant increases in the frequency or severity of incidents or non-serious incidents that may require reporting under Article 88 of Regulation (EU) 2017/745.

SERIOUS INCIDENT: Any incident that directly or indirectly led, might have led or might lead to the death of a patient, user or other person, the temporary or permanent serious deterioration of a patient's, user's or other person's state of health, or a serious public health threat, as defined in Article 2(65) of Regulation (EU) 2017/745.

- ABBREVIATIONS

CAPA – Corrective and Preventive Action

FMEA - Failure Modes and Effects Analysis

MR – Management Representative

NCR – Nonconformance Report

PR – Protocol

TR – Test Report

- RESPONSIBILITIES

The Management Representative (MR) is responsible for maintaining this procedure in a current state, and for ensuring that records of the Testing, Measurement and Analysis are available to Top Management and to overall staff where appropriate. The results of the analysis maybe used to determine if any Corrective Actions are necessary (see QSP 8.5-1 Corrective and Preventive Action). The MR will maintain an electronic Protocol/Test Report Log, assigning the PR and TR # (e.g. TR 14-001 Rev 0).

The Production Manager (PM) is responsible for ensuring Protocols and Test Reports that apply to design, development and production are written and approved. Protocols must be approved prior to the start of any testing. Where appropriate, standing Protocols may be established for periodic testing, such as pull tests. The PM will ensure Test Reports covering the data collected for these standard Protocols is periodically updated and presented to management. The PM will obtain Protocol and Test Report numbers from the MR.

- PROCEDURE

  - Identify Requirement

    - There must be a business need for the information. Clearly identify what is trying to be achieved.

    - If the collected process data cannot be used to improve and/or monitor the product quality or the process efficiency, then consider discontinuing the collection of that information.

    - Examples of requirements are the percentage of accepted product from a particular supplier, biocompatibility testing for product changes and the percent of complaints based on units sold.

Note: All Quality Objectives must be measurable and analyzed to ensure that plans are achievable.

  - Identify Required Information

    - Refine the requirement to specifically what testing and information is required. Describe the breadth of the testing, including products or designs to be covered.

    - An example related to the supplier acceptance rate of the products received would be the acceptance rate (percentage of acceptable product shipments compared to total shipments received).

  - Identify Method of Testing and Data Collection

    - Determine the Who, When, Where and How of testing and data collection.

    - If necessary, perform trials with the appropriate personnel to determine the degree of instructions necessary and then implement any required actions.

    - Develop a sampling plan based on product severity (refer to QSP 8.4-3).

  - Collect Information

    - Clearly define the information required for the testing, measurement and analysis (part lot number is always required).

    - Collect information in the normal course of performing production and support activities.

    - Data is continually collected via appropriate logs. Ex. Customer Feedback / Complaints QSP 8.2-2, Nonconformance QSP 8.3-1

      - Nonconformity data is then collected via the nonconformance log and monitored using trend charts, which are reviewed during Management Reviews to identify patterns and guide corrective actions.

    - QMS Risk Management QSP 5.6-2

      - Create a Risk Management file

    - Risk Management QSP 7.3-3

      - Use FMEA to provide identification and traceability for each hazard to the risk evaluation

    - Product inspection QSP 7.4-2 data is collected through discrepancy nonconformity

      - Receiving will be conducted in accordance with RFD-009 Sample Size

      - All final products receive 100% inspection

    - Process Validation QSP 7.4-5

      - Use Verify / Validate Process

    - Post Market Surveillance QSP 9.1-7

      - A system to proactively collect and review experience gained from devices

  - Chart Information

Data results may be graphed for ease of interpretation and analysis as needed.

  - Analyze Information

    - Periodically review the collected information to determine that no unsatisfactory trends are developing or that a process required additional process controls to improve the process.

    - In some cases, the information could be analyzed daily; in others (Complaints, CAPAs, Audit Results, NCRs, etc.) the period may be monthly, quarterly, semi-annually or annually, depending on whether any issues are being tracked and whether the processes are considered as satisfactory or requiring improvement. For testing, the analysis should be documented.

    - All parameters measured and analyzed are reported and reviewed annually at the Management Review Meeting (see QSP 5.6-1 Management Review).

    - Results are compared to the established Quality Objectives.

  - Satisfactory

    - Accepted results for any testing or measurements should be clearly defined by the MR and documented upon completion.

    - If the acceptance criteria are not achieved, corrective actions should be developed and appropriate changes made to the Protocol, if applicable. It should be read and approved by the Management Representative (MR), that the standards being achieved with the acceptance criteria are changed. Any deviations from the Protocol must be clearly documented with the rationale for deviation.

  - Raise CAPA to Track Improvements / Test Results

    - When appropriate, for any noted issue or unsatisfactory trend line, a CAPA should be raised as the preferred manner of tracking improvements. Refer to QSP 8.5-1 Corrective and Preventive Action.

    - If appropriate, the CAPA can refer to an improvement plan, which has dedicated resources and a timeline that the improvements are addressed, tracked and analyzed.

    - If the issue is not resolved, then changes should be incorporated and new information collected and analyzed.

    - Continue the refinement of the corrective action until the problem has been resolved, or all avenues have been exhausted, and then a management decision can be made to make a change in technology or live with the issue by controlling it in some appropriate manner.

    - The CAPAs form records of the Improvement process at work.

    - For testing, document test results using the format of the. Recommended actions from the testing may be tracked using a CAPA or other appropriate methods.

    - The analysis of data shall include data generated as a result of monitoring and measurement and from other relevant sources and include, at a minimum, input from:

      - Conformity to product requirements

      - Characteristics and trends of processes and product, including opportunities for improvement

      - Suppliers

      - Audits

- FLOWCHART