Tulip Medical

Quality System Procedure
QSP 8.5-1: Corrective and Preventive Action

Approval

V.P. Regulatory Affairs 	Date

Regulatory Affairs Coordinator						Date

Document Control Administrator 	Date

| Rev | Date | Description of Change |
| --- | --- | --- |
| 0 | 05/07/2013 | Release documentation – Implementation of document for compliance to FDA QSP Subpart J 820.100 and ISO 13485 8.5. |
| 1 | 09/23/2014 | Updated reference to ISO 13485:2012 and changed CAPA log to a single log sheet. |
| 2 | 06/28/2018 | Updated Flowchart and aligned with Complaint procedure. |
| 3 | 08/29/2018 | Updated Section 7.0 Procedure with the inclusion of 7.5.1. |
| 4 | 09/10/2018 | Updated the following information: Sec. 2: Included CAPA information sources and documentation method Sec. 3: update the list Sec. 7: Rewrite this section to include safety assessment and reporting requirement. Root/Cause analysis, CAPA verification and effectiveness requirement. Define common method for conducting root/cause analysis. Added CAPA timeline guidelines and records retention. |
| 5 | 07/29/2019 | Section 7.1.2 added ‘When determining if a Compliant should open a CAPA, refer to QSP 8.2-2 Customer Feedback & Complaints, section 7.0 Procedure. For Complaints, the Feedback Team will make a determination based on risk and other factors such as trends, if a CAPA should be started or not.’ |
| 6 | 04/14/2020 | 1.0 Purpose: added per the Quality Manual requirements; added bullet points 2.0 Scope: added ‘1’ to QM 3.0 Reference Documents: added ‘1” to QM 6.0 Responsibilities & Authority: added ‘& Authority’; added CAPA Owners 7.0 Procedure, 7.1: added 7.1.3.3 and 7.1.3.4 7.2 Safety Assessment: deleted the first paragraph 7.3.2 added ‘(Form 8.5-1-2) 7.3.3 Root Cause(s) Description: added ‘/Root Cause Statement’; 7.3.3.1 added ‘on the CAPA form (Form 8.5-1-2)’ to end of paragraph 7.5 CAPA Verification of Effectiveness, 7.5.1.2 added ‘verifying or validating…’ to end of paragraph |
| 7 | 03/27/2024 | Reformatted QSP 2.0 Added alternative method for Product Complaint Reports 3.0 Removed Form 8.5-1-4 -Added QSP 7.3-1 5.0 Added DCA to abbreviation 7.0 Updated subparagraph headers 7.2 Updated Initiation process 7.3 Clarified risk evaluation 7.7 Added Identifying Potential New Hazard Analysis 7.8.1 Added the Verification of Effectiveness (VoE) 7.10 Updated Timeline Chart -Added clarity in justifying delays and reviewing them in the annual Management Review. 9.0 Updated Flowchart |
| 8 | 04/11/2025 | 4.0 Added Correction to provide definition -Updated definition of Corrective Action 7.6.1 Added Correction to be part of the “Corrective and/or Preventive Action plan” |
| 9 | 5/23/2025 | Aligned CAPA effectiveness and risk analysis sections with EU MDR 2017/745 by referencing Annex I General Safety and Performance Requirements (GSPR). |

- PURPOSE

To assign responsibility, designate authority and describe the system for corrective and preventive actions to prevent future occurrences of nonconformity, or to make improvements to the quality system per the Quality Manual requirements.

To establish a process for developing and implementing appropriate corrective and preventive actions when a process or product non-conformance, or other undesirable situations, is observed.

The Tulip Medical CAPA process is central to maintaining the effectiveness of the management system and the quality and safety of our products.

- SCOPE

This procedure applies to all actual or potential nonconformities that affect Tulip Medical Products.

CAPA information sources and documentation methods include, but are not limited to, those shown below. Although the CAPA Record is the most common documentation method, alternative methods are permitted when effectiveness follow-up is ensured by general quality system controls.

| CAPA Information Source | Associated Document | CAPA Documentation Method | Method for Effectiveness Verification/Validation |
| --- | --- | --- | --- |
| Post Market Surveillance Reports | QSP 9.1-7 | CAPA Record | CAPA Record |
|  |  | Post Market Surveillance Report | Ongoing post market surveillance activities |
| Non-Conformance Reports | QSP 8.3-1 | CAPA Record | CAPA Record |
|  |  | NCR, Document Change Notice | Ongoing product inspections |
| Product Complaint Reports | QSP 8.2-2 | CAPA Record | CAPA Record |
|  |  | Complaint Investigation | Next Meeting Agenda |
| Quality Audit Reports | QSP 8.2-1 | CAPA Record | CAPA Record |
|  |  | Audit Response | Re-audit of actions stemming from previous audit |
| Quality Manual Management Review Meetings | QM 1 | CAPA Record | CAPA Record |
|  |  | Meeting Minutes | Next Meeting Agenda |
| Supplier Performance Reports | QSP 7.4-1 | CAPA Record | CAPA Record |
|  |  | Supplier Performance Report | Ongoing supplier performance monitoring |
| Design History Files | QSP 7.3-1 | CAPA Record | CAPA Record |
|  |  | Design History File | Follow-up at subsequent design reviews |

- REFERENCE DOCUMENTS

| Form 8.5-1-1 | CAPA Log |
| --- | --- |
| Form 8.5-1-2 | Corrective and Preventive Action |
| QM 1 | Quality Manual |
| QSP 4.2-2 | Control of Quality Records |
| QSP 5.6-1 | Management Review |
| QSP 7.3-1 | Design & Development |
| QSP 7.4-1 | Purchasing and Supplier Evaluation |
| QSP 8.2-1 | Quality Audits |
| QSP 8.2-2 | Customer Feedback Complaints |
| QSP 8.3-1 | Control of Nonconforming Product |
| QSP 9.1-7 | Post Market Surveillance |

- DEFINITIONS

Correction - immediate action taken to eliminate a detected nonconformity.

Corrective Action – a systematic process to eliminate the cause of a detected nonconformity or other undesirable situation to prevent recurrence

Management Representative – The member of Tulip Medical whose cross-functional role encompasses the development and monitoring of the quality management system to ensure accuracy and effectiveness of all Quality System Procedures (QSPs).

Nonconformance – Non-fulfillment of a specified requirement.

Preventive Action – Action taken to eliminate the causes of a potential nonconformity, defect or other undesirable situation in order to prevent occurrence. The preventive action should eliminate or prevent the occurrence of the potential issue.

Root Cause Analysis – The analysis necessary to determine the original or true cause of a system, product or process nonconformity. This effort extends beyond the effects of a problem to discover its most fundamental cause.

- ABBREVIATIONS

CAPA – Corrective and Preventive Action

MR – Management Representative

QA – Quality Assurance

DCA – Document Control Administrator

- RESPONSIBILITIES and AUTHORITY

Management Representative (MR) - is responsible for maintaining this procedure in a current state as well as ensuring that the Corrective and Preventive Action program is efficient and effective. All personnel assist in carrying out these responsibilities.

Quality Assurance (QA) - is responsible for assuring CAPAs are followed to closure.

CAPA Owners - ensure that the corrective actions, to resolve nonconformities, are completed and implemented without undue delay.

Note:	Anyone in the company has the authority and responsibility to start a Corrective and Preventive Action (CAPA). If they have not been trained, they are to convey this information to someone trained in this procedure.

- PROCEDURE

  - CAPAs may be originated from a variety of internal or external sources, to include but not limited to;

  - Internal Audit

  - External Audit

  - Nonconformities

  - Complaints

  - Management Review

  - Risk Management

  - Corrective Action and Preventive Action (CAPA) Initiation

    - All CAPA numbers are to be requested from the DCA

  - The number will be the next in sequence from the CAPA Log Form 8.5-1-1

    - Commence Form 8.5-1-2 Corrective and Preventive Action and provide a concise, clear problem statement summary

    - From the list provided select the best source of information that originated the CAPA.

  - When the source is not one of the options provided, check other and provide where the information was sourced.

    - Based on the description given on the CAPA select whether it is product related

  - If it is product related, a specification must be provided

    - All CAPA forms must indicate if it is being initiated due to a Corrective Action or as a Preventative Action

  - Risk Analysis / Safety Assessment

    - Verify if the risk assessment has been identified in either the Complaint or Nonconformance.

  - If no risk has been identified, refer to RSK-04, RSK-07, RSK-08, RSK-09, RSK-11 for Risk Evaluation.

    - Based on the described observation, RA/QA will determine if it represents a safety risk and if it has any regulatory impact – refer to QSP 8.2-2 and ensure evaluation is in alignment with MDR 2017/745 Annex I, Chapter I, Section 3: Risk Management.

  - Root Cause Analysis

    - Determine the Root Cause of the problem. The Root Cause is different from the symptom (the immediate problem) in that when the Root Cause of the problem is fixed, the problem will not recur. In the case of a preventive action, fixing the root cause ensures the problem will not happen.

  - Use The Five Why’s. An approach where ‘why’ is asked multiple times until the root cause is uncovered.

  - Root Cause Summary

    - After the root cause investigation has been performed; describe the root cause(s) contributing to the nonconformance that justifies the need for corrective or preventive action.

  - Corrective and/or Preventive Action plan

    - The CAPA Owner shall develop an appropriate correction and corrective action plan, or preventive action plan based upon root cause determination. The proposed action plan (with a list of specific deliverables and due dates) is submitted for approval.

    - Corrective and Preventive actions should be implemented in a timely manner. The owner will provide completion dates, based on the complexity and scope of the implementation activities and Tulip Medical Product guidelines.

    - Any change to the actions and extensions for the Corrective and Preventive Actions due dates must be documented and approved.

  - Identifying Potential New Hazard Analysis

    - After the CAPA has been implemented identify if the changes caused a new hazard

  - If a new hazard was identified, update and attach Risk Analysis containing the new hazard, analysis, and mitigation

  - CAPA Verification of Effectiveness

    - The Verification of Effectiveness (VoE) shall include a review of applicable documents to confirm that all necessary updates (e.g., procedures, forms, work instructions) have been identified, completed, and controlled. Evidence of document updates shall be attached to the CAPA form (Form 8.5-1-2).

    - Quality Assurance shall ensure the effectiveness of the CAPA is verified.

    - The CAPA Effectiveness Monitoring Results shall include the verification of the corrective/preventive actions to ensure that the actions were effective and did not adversely impact the product or process involved to meet safety and performance requirements and to meet applicable regulatory requirements, including those outlined in EU MDR 2017/745 Annex I, General Safety and Performance Requirements (GSPR). Examples of methods used to verify the effectiveness of corrective and preventive actions taken are Control Charts or Trend Charts, internal audit results, etc.

    - Records pertaining to the verification of effectiveness of the CAPA shall be maintained in the CAPA file (i.e. include a copy of the control chart, training records, etc.).

    - Reporting on the results of this process is an integral part of Management Review per QSP 5.6-1.

  - CAPA Approval & Closure

    - To close a CAPA file, evidence shall be retained to demonstrate that all corrective/preventive actions were completed.

    - No file will be closed until all actions are satisfactorily completed.

    - Documented evidence of the completed action items will be maintained with the CAPA file.

    - Quality Assurance will review and ensure the CAPA is completed and closes the document.

  - CAPA Timeline Guidelines

    - QA ensures that CAPAs are processed in accordance with the following timeliness guidelines unless otherwise justified by the CAPA.

| Task (Completion of CAPA Section) | Timeliness Guideline |
| --- | --- |
| Initial Review & Action Assignment | Within 5 working days of date initiated |
| Root Cause Analysis | Within 30 calendar days of CAPA number assignment |
| Corrective Action Implementation | Per Action Plan ‘Estimated Completion Date’ + 1 month |
| Verification of Effectiveness | Within 12 months - Anything over 12 months requires a justification |

    - Justification for delays must include a documented explanation of the cause and an updated timeline for task completion. Examples include additional time required for effectiveness monitoring, external constraints, or unanticipated complexities in implementation approved by the head of Quality/MR.

    - CAPA delays and their justifications will be reviewed during management reviews to ensure ongoing compliance with the timeliness guidelines and effectiveness of the CAPA system per QSP 5.6-1.

- RECORDS AND RETENTION

The original CAPA and all supporting documentation (including any validation and verification) shall be held in the CAPA Files in Quality Assurance and will be maintained in Document Control for the record retention period specified in QSP 4.2-2.

- PROCESS FLOWCHART